Optimal Control Techniques applied to the mechanical system

Thesis of the project for the Optimal Control course

Professor:                                                                                                                                     Student:

Domenico Famularo                                                                                                Rustemova Aynura

Matricola:234277

Academic year 2021/2022

Index

Summary..................................................................................................................................

Chapter 1..................................................................................................................................

1 1 Mathematical Model..........................................................................................................

1.2 Linearization ......................................................................................................................

1.2.1Equilibrium...........................................................................................................

1.2.2 Calculation of the linearized system....................................................................

1.3 Stability ................................................ .............................................................................

1.4 Structural Properties .........................................................................................................

1.4.1 Reachability .........................................................................................................

1.4.2 Observability .......................................................................................................

Chapter 2 ................................................ ................................................................................

2.1 Finite Time Horizon Linear Quadratic (LQ) Optimal Control - Discrete Time Case …….……

2.2 Infinite Time Horizon Discrete Time Linear Quadratic Control..........................................

2.3 Linear-Quadratic tracking problem ....................................................................................

2.3.1 LQ tracking over finite time horizon.....................................................................

2.3.2 LQ tracking with integral control……………………………………………………………………..

2.4 LMI formulation of Linear-Quadratic control .....................................................................

2.4.1 LMI for discrete time LQ control………………………………………………………………………

2.5 Constant disturbance rejection with integral control………………………………………………………

2.5.1 Disturbance Rejection and Position Tracking…………………………………………………….

Conclusions ...............................................................................................................................

Bibliography ..............................................................................................................................

Summary

The purpose of the following work is to analyze the behavior of a nonlinear system formed by a platform connected to a horizontal axis through a torsional spring and to determine, through the techniques learned on the subject, control laws that make the system stable in an optimal way.

Keeping in mind this goal, the project was divided into two basic phases:

• Modeling, linearization and analysis of structural properties.

• Application of optimal control techniques.

In the first phase, after having modeled the system in a software environment, the non-linear system is linearized around equilibrium from which its stability and structural properties of reachability and observability are analyzed. Then, in the second phase of the project, the optimal control techniques that have been taught on the subject are applied, which are:

• Linear-Quadratic Control (LQ) in discrete time on a finite and infinite time horizon.

• Discrete time LQ tracking problems on a finite time horizon and disturbance rejection with integrator

• Linear Matrix Inequality (LMI) formulation of the LQ control.

To do this, a script was created in Matlab and used the toolbox Simulink to model the system and schemes.

Chapter 1

Modeling, Linearization and Structural Properties

1.1 Mathematical Model

Consider the mechanical system shown in Figure 1.1,

Figure 1.1 Mechanical system

It consists of a platform able to rotate around a horizontal axis on which a mass slide. The angle of rotation of the platform with respect to the horizontal plane is indicated with θ and distance of the mass from the axis of rotation is x. The platform is connected to the rotation axis through a torsional spring with elastic constant k. The mass is free to slide on the platform remaining connected to the rotation axis through spring with elastic constant h and zero rest length. The associated dynamical model is described by the following system of nonlinear equations:

Where J is the moment of inertia associated with the platform, Ca is the coefficient of friction and g is gravitational acceleration.

τ is considered as actuator, while F is regarded as disturbance. The numerical values for parameters are following:

m=0.15kg, J=750kgm2, Ca=1.5Nsm, g=9.81ms2

To represent the system in state space, the following variables are chosen:

x=xθxθ=x1x2x3x4,  u=τF=u1u2

State-space representation of the model:

x=fx,u=x3x4-mgsinθ-Cax3+mx1x42-hx1+u2m-kx2-mx1gcosθ+2x3x4-u1J+mx12

1.2 Linearization

Real dynamical systems are never perfectly linear, but can be approximated to any predetermined movement (such as, for example, an equilibrium point) by means of suitable linear models, called linearized models. One of the main advantages of linearization is that simpler, more powerful and more numerous methodologies are available for the analysis and control of linear dynamic systems than in the non-linear case.

1.2.1 Equilibrium

The experimental setup is fixed with respect to x=0, θ=0 under the hypothesis that disturbance F=0. We must derive constant torque τ in equilibrium fashion, which is capable of realizing the equilibrium x=0, θ=0,F=0. This task is done using Simulink tool which is called model linearizer.

The values of displacement x and angle θ are kept constant to 0.

The value of disturbance which is the second row of the input matrix is set to 0.

Having trimmed the model, we obtain operating point which contains the equilibrium of the system:

x=0000, u=10-100.52150.

1.2.2 Calculation of the linearized system

We linearize the system around the equilibrium utilizing the function linmod and obtain the following matrices:

Ac=00100001-0.666-0.0019-9.809-0.0066-10000, Bc0=00000-0.00136.6660,

Cc=1000010000001001, Dc0=00000000

We must consider only the input columns related to the actuators, so matrices Bc0 and Dc0 will be transformed into the following matrices:

Bc=000-0.0013, Dc=0000

1.3 Stability

The stability of a system can be thought of as a continuity in its dynamic behavior. If there is a small change in the inputs or initial conditions, a stable system will exhibit small changes in its disturbed response. On the other hand, in an unstable system, any disturbance, no matter how small, will cause states or outputs to grow without limit or until the system burns out, disintegrates or becomes saturated. Therefore, stability is a fundamental requirement of dynamic systems intended to perform process operations or signals, and it is the first thing that must be guaranteed in the design of a control system.

The criterion chosen to verify the stability of the treated system is the Reduced Lyapunov criterion, which in the continuous case is based on the following statements:

1. If the matrix has all stable eigenvalues, that is Reλ<0, then the equilibrium is locally asymptotically stable.

2. If the matrix has at least one unstable eigenvalue, that is Reλ>0, then the equilibrium is unstable.

3. If at least one eigenvalue of the matrix is on the imaginary axis, that is Reλ=0, then the criterion fails to conclude anything.

At this point, to verify the asymptotic stability, it is necessary to calculate the eigenvalues of the matrix Ac on MATLAB using the command eig resulting:

λA=-9.9329 + 0.0000i-0.0735 + 0.1150i-0.0735 - 0.1150i0.0800 + 0.0000i

Clearly, the equilibrium point is unstable. In this case the controller has to perform two tasks:

- Fulfill the LQ requirement, which is to regulate the transient and minimize the energy

- Stabilize the operating point

1.4 Structural Properties

In order to achieve the design goal, a fundamental step is to analyze the structural properties of the dynamic system. In particular, attention must be paid to the reachability and observability of the system. Reachability is an input-state property linked to the control law, that is, to the position of the actuators inside the plant, so as to obtain the desired results from it. While observability is closely linked to the sensor side.

1.4.1 Reachability

The reachability property describes the possibility of modifying the system state starting from a particular pre-established initial state by acting appropriately on the input. Therefore, the concept of reachability is related to the effective possibility of being able to guide a given system, through its inputs, to assume conditions arbitrary. The reachability property is related to matrices A and B, in fact the reachability matrix can be calculated as follows:

R=BABA2B…An-1B

In MATLAB it is possible to calculate it through the command ctrb which naturally receives matrix as parameters A and B of the linearized system. The reachability matrix of the system in this case is:

R=0000.01310-0.0013000-0.0013000.01310-0.13080

The reachability condition is as follows:

“A linear system is completely reachable if and only if its reachability matrix has maximum rank, i.e. rank equal to n".

rankR=rankBABA2B…An-1B=4

To check if this happens in the system model, we used the command rank on MATLAB and it was obtained that rankR=4, which means that it has a full rank, and in this way the condition of complete reachability is satisfied.

1.4.2 Observability

The observability property describes the possibility of estimating the initial state of the system by measuring the output (∙) and the entrance (·) over a given time interval, and is linked to matrices A and C, in fact the observability matrix can be calculated as follows:

θ=CCACA2::CAn-1

In MATLAB it is calculated by function obsv which takes arrays as parameters A and C of the linearized system.

Similar to reachability, the condition of complete observability establishes that a linear system is completely observable if the matrix has full rank, that is, equal to n.

rankθ=rankCCACA2::CAn-1=4

This allows us to state that the system is completely observable.

Chapter 2

Application of Optimal Control Techniques

2.1 Finite Time Horizon Linear Quadratic (LQ) Optimal Control - Discrete Time Case

Considering the linear and stationary discrete time model

xk+1=Axk+Buk

with initial state x0=x0 known. The sequence of control moves

u0,u1…un-1=u0,N

such that the cost

Jx0,u0,N=k=0N-1xkTQxk+ukTRuk+2ukTMxk+xNTSxN

said LQ cost, is minimal, and where

- The penalty function xNTSxN=ψ(xN), denotes a distance measure of some terminal state components with respect to the desired equilibrium 0x.

- xkTQxk+ukTRuk+2ukTMxk=lk,xk,uk is called the loss function:

- xkTQxk, is a quadratic form measuring the distance of the intermediate states xk with respect to 0x and makes it possible to evaluate in an indirect fashion, the transient behavior of the regulated state plant (or a part of it);

- ukTRuk, is a quadratic form, measuring the control effort necessary to drive state plant from the initial to the terminal value;

- ukTMxk, is a mixed term representing energy/power transfer phenomena between the plant and the outside environment.

So, the optimal cost value

minu0,NJx0,u0,N=x0TP0x0

depends on the shaping matrix Pi, positive semidefinite, which is equivalent to the solution of the Riccati difference equation:

Pi=Q+ATPi+1A-ATPi+1BR+BTPi+1B-1BTPi+1A,  0≤i≤N-1

Then, the optimal control move is a linear feedback of the time-varying state:

ui=-Fixi

where  Fi=R+BTPi+1B-1BTPi+1A is called Kalman Gain.

To derive the values of Fi and Pi we make use of the so-called Backwards Iterations, calculated in the MATLAB script, in order to do this, we need to choose the weight matrices of the LQ cost: S,Q and R:

- S is equal to CNTCN, where is  CN∈Rl×nx, denotes a fake output matrix which selects the terminal state vector components we are interested to measure with respect to the equilibrium.

- Q and R they are obtained starting from a fictitious output

zk=Czxk+Dzuuk

also called “performance output” with nz=nx+nu components, Q=CzTCz and R=DzuTDzu. The entries of Cz identify which states we are interested to weight during transient; the entries of Dzu characterize the way the actuators are used to generate the control effort term by means of the positive parameter ρ; Cz∈ Rnz×nxis organized in blocks by rows, which must be orthogonal to the columns of Dzu∈ Rnz×nu so that there are no mixed terms in the cost (M=0).

Then, we are interested to weight position x and angle θ:

Cz=100001000000

Dzu=00ρ

In this case, the choice of ρ  is based on the physical characteristics of the system and its moving components, a very fast transient that could cause structural damage is not desired. In particular, if we want faster transient, it is necessary to change the value of ρ, which influences the efficiency of the actuators that make the control "strong" if R increases or "mild" if R decreases.

At this point it is necessary to find the sampling time and the number of steps of the LQ algorithm.

In order to calculate sampling time, unstable eigenvalue is ruled out and the nearest to imaginary axis eigenvalue is chosen, the natural frequency of which is considered as an estimate of bandwidth of the plant.

ωBW=abs-0.0735 + 0.1150i=0.1365

The sampling time must satisfy the following inequality:

Ts≤2π20ωBW,  Ts≤2.3s

So,  Ts=0.5s is chosen.

The final time is fixed as = 30.

The number of steps is evaluated as the division between the final time and the sampling time

N=tfTs

We now proceed to calculate the system discretized by the linear model and the sampling time chosen, which results:

A=0.9417-0.86080.0948-0.3960-0.00100.9969-0.00010.9989-0.0624-0.0019-0.9273-0.0059-0.0063-0.0002-0.86080.9969, B=0.0002-0.0007 0.0005-0.0013

C=1000010000001001, D=0000

Figure 2.1 Response of the feedback system with finite time horizon LQ

Figure shows the result obtained from the simulation of the system, which states are perturbed by the values dx0 = [0.1;0.1;0.1;0.1]. It is evident that, despite these perturbations, the system returns to its state of equilibrium, which corroborates the accuracy of the parameters calculated for the LQ control.

2.2 Infinite Time Horizon Discrete Time Linear Quadratic Control

For the Linear-Quadratic control discrete time on an infinite time horizon, that is, when

N → ∞, the model is considered with the addition of the performance output

xk+1=Axk+Bukzk=Czxk+Dzuuk

And it's supposed (A, B) is stabilizable and (A,Cz) detectable, then, on the cost, it is assumed S=0:

k=0∞xkTQxk+ukTRuk=k=0∞zkTzk

So, the law that minimizes it is a stabilizing state feedback:

uk,∞=-F∞xk

Where F∞ depends on P∞, which is limit for N → ∞ is the solution of the Riccati difference equation. The LQ cost in correspondence of uk,∞ is equal

x0TPx0

On MATLAB, the optimal gain and Riccati solution are calculated with the command lqr, then there is no need to recalculate the weight matrices and simply recycle them. Once this is done, the following results are obtained:

P∞=7.0064-11.88180.6038-11.7460-11.881840.9512-1.1950 53.04140.6038-11.7460-1.195053.04140.0607-1.1804-1.1804109.5547

F∞=[60.1336  -290.2118  6.0417 -645.0078]

This constant Kalman gain is equal to the first row of Backward iterations:

F1=60.1336-290.21176.0417-645.0076

The equivalence is described by the following graph, depicting the same behavior as in case of finite time horizon.

Figure 2.2 Response of the feedback system with infinite time horizon LQ

2.3 Linear-Quadratic tracking problem

Let us consider a discrete time LTI dynamical system endowed with an output (not to be confused with the performance output)

xk+1=Axk+Buk

yk=Cxk

an optimal finite time horizon tracking problem consists in the determination of an input sequence u[0,N) such that the performance index

k=0N-1(yk-rk)kTQ(yk-rk)k+ukTRuk+(yk-rk)NTS(yk-rk)N

is minimum. We will suppose that Qy>0, 𝑅 >0, Sy>0. Matrices Qy are Sy are positive definite since, in a tracking problem, we are interested in weighting all the plant outputs, no exception. The signal r[0,N) is the reference a priori known or available. A tracking problem can be restated, in some sense, as a regulation problem by considering the following variable εk = yk-rk  which is the tracking error and the performance becomes

k=0N-1ξkTQξk+ukTRuk+ξNTSξN

2.3.1 LQ tracking over finite time horizon

The servo problem

If the reference rk is obtained as a free response of a dynamical system named exosystem or reference generator

xr,k+1=Arxr,k

rk=Crxr,k

which means that the reference is obtained as a linear combination from some free modes. In particular if some of the eigenvalues are located on the unit circle then it is possible to obtain step and/or periodical signals. The tracking performance index can then be written as

k=0N-1xkTxr,kTCT-CrTQyC-Crxkxr,k+ukTRuk+xNTxr,NTCT-CrT SyC-CrxNxr,N

and, by defining an augmented state

ξk=xkxr,k∈Rnx+nr

we obtain a fake LTI model, instrumental for our purposes

ξk+1=Aξk+Buk

where,

A=A00Ar, B=B0

and the performance index becomes an LQ regulation problem

k=0N-1ξkTQξk+ukTRuk+ξNTSξN

with

Q=CT-CrTQyC-Cr

𝓢=CT-CrTSyC-Cr

So, in the augmented state, Riccati iterations are defined

PN=SPk=Q+ATPk+1A-ATPk+1BR+BTPk+1B-1BTPk+1A,  0≤k≤N-1

to generate the optimal control moves, which will be a time-varying feedback

uk=-Fkξk

where Fk is the Kalman gain of the augmented state.

Then the control law can be decomposed into the sum of two components:

Fk=FkFkv

with Fk∈Rnu×nx, Fkv∈Rnu×nr

uk=-Fkξk=FkFkvxkxr,k=-Fkxk-Fkvxr,k

the first term is a standard state feedback law (effective regulation) whereas the second, which depends on the reference generator is the feedforward component. The feedback component term is in charge to regulate the time behavior of the tracking error εk, 𝑘 = 0,1…𝑁 with respect to perturbations related to the initial model state x0 and initial discrepancy between the reference and the output to be tracked. The feedforward terms have the task to fix the shape of the output yk as near as possible, once the transient on the tracking error is over, to the reference rk.

-Fkv

Σr

Σ

xr,k                -Fkvxr,k +                                  yk

+

xk

-Fkxk

-Fk

Figure 2.3 Servo problem, feedback and feedforward components

We want to track position x angle θ.

Firstly, we consider an angle and therefore Cy=[0 1 0 0], a reference signal is xr,k=π18.

In order to generate the exosystem we start from the continuous time counterpart:

xrt=Ar,cxrtrt=Cyxrt

When Ar,c=0 we obtain Ar=eAr,cTs=1 and Cr=1.

Feedback and feedforward gains can be derived by considering the structure of the augmented state plant:

A=A00Ar, B=B0

A=0.9417 -0.86080.0948-0.39600-0.00100.9969-0.00010.9989 0-0.0624-0.0019 0 -0.9273-0.00590-0.0063 -0.00020-0.8608  0.9969 0001, B=0.0002 -0.00070.0005-0.00130

and the related Q and S matrices in the performance index:

Q=CT-CrTQyC-Cr

𝓢=CT-CrTSyC-Cr

The weight matrices of the LQ cost are calculated in the augmented form with Qy=Sy = 1.

Q=S=1 000-10000 000 -1 0000 000  0 0001

Since, the optimal Kalman gain converges towards the constant value, it is possible to implement suboptimal strategy, which is done by taking the first row as F and discarding other rows.

The value of tuning parameter ρ=0.0001 to make the signal as close as possible to reference signal.

Figure 2.4 Angle tracking

As can be seen from the graph, there is a slight steady-state error.

Next, we track the position Cy=[ 1 0 0 0] applying the same technique with reference signal xr,k=0.1 and tuning parameter ρ=0.01

Figure 2.5 Position tracking

A marginal steady-state error on position can be observed.

In both cases we are forced to move tuning parameter to lower value in order to decrease steady-state error, paying price for the voltage whose behaviour can become unacceptable.

2.3.2 LQ tracking with integral control

If the reference signal is a step whose amplitude is 𝑟 (setpoint tracking) it is possible to design a feedback control strategy such that the tracking error asymptotically goes to zero

limk→∞yk=r

Within a state space approach this can be accomplished by considering the following augmented state space model:

xk+1=Axk+Buk-1+B∆ukuk=uk-1+∆uk

the plant input uk is obtained by the incremental quantity uk=uk-1+∆uk which denotes the actuator signal exploited to design the control strategy. The input uk-1 becomes an additional state and we will retake the previous section servo approach with Ar = 1. The introduction of the incremental input ∆uk is instrumental to zeroing the tracking error which conversely will be asymptotically finite but not zero. The feedback-feedforward decomposition of the control law holds true and we will obtain:

∆uk=-Fkξk=Fk     Fu,kFkvxkuk-1xr,k=-Fkxk-Fu,kuk-1-Fkvxr,k

Feedback and feedforward gains can be derived by considering the structure of the augmented state plant:

A0=A00B000Inu B0=BInu

A=A000Ar, B=B00

and the related Q and S matrices in the performance index:

Q=CT0nu×nr-CrTQyC0nr×nu-Cr,      𝓢=CT0nu×nr-CrTSyC0nr×nu-Cr

zz-1

Σr

-Fkv

Σ

∆uk                            uk                       yk

+

+      +

-Fk

z-1

-Fu

Figure 2.6 Reference tracking with integrator

Figure 2.7 Tracking the angle with integrator

Figure 2.8 Tracking the position with integrator

With this technique, we are able to reach zero steady-state error without tuning ρ to small values. If we tune ρ in this type of strategy, only the transient behaviour is changed.

2.4 LMI formulation of Linear-Quadratic control

Linear Matrix Inequality (LMI) has an important application in control theory by reducing problems such as

1. Asymptotic Stability.

2. Summary of the control law  u=-Fx

3. Solution of the LQ control problem over an infinite time horizon.

in the computation of the solution to transform it into a convex optimization problem that can be solved numerically through semidefinite programming.

Now we will analyze this formulation in solving LQ optimal control problems.

2.4.1 LMI for discrete time LQ control

Assigned the dynamic system:
xk+1=Axk+Buk

yk=Cxk

The goal is to find the law of control: 
uk=-Foptxk

such that

k=0∞zkTzk

is minimal. Obviously, we must consider the knowledge of the initial state and hypothesize that the system is stabilizable and detectable and the states measurable.

Then, the optimization problem that numerically determines the solution to this LQ control problem on an infinite time horizon in discrete time has the form:

minY,k,ααx0TY-1x0≤1Y>0YYAT-KTBTYCzT-KTDzuTAY-BKY0CzY-DzuK0αInz≥0

with K=FY, X= αY-1 and called slack variable, which is reached after using variable substitutions, Schur's complements and congruence transformations that render the constraints in LMI.

Once the LMI variables are defined α,K,Y as well as the alleys of the upper bound, the positivity on the and Riccati inequality in discrete time, we proceed to optimize using the YALMIP toolbox with the lmilab solver, checking that the problem is feasible and ultimately obtaining the gain:

F=KY-1=[46.6940  -240.8937  4.6862 -577.3760]

This result actually coincides with the one obtained through the Riccati iterations in closed form, verifying the equivalence between the two methods.

Figure 2.9 Response of the system with LMI

2.5 Constant disturbance rejection with integral control

In this section we want to reject a constant disturbance Fd on the first state variable (position/linear displacement). We have linearized model Σ and a standard state-feedback law is used to find steady state gain  -F, described by the following diagram

dk

uk

Σ

+		    yk

+

-Fq

-F

xk

qk                   yk

Figure 2.10 Disturbance rejection with integral control

where dk is discretized version of step with amplitude D.

In general, limk→∞yk=0, but due to constant exogenous signal  dk, yk converges to constant quality, which represents error yk→(yd≠0). We want to cope indirectly with this type of error by reducing this quantity as much as possible by performing feedback with integrator such that the following system is asymptotically stable:

xk+1=Axk+Buk+Bkdkqk+1=qk+yk

Control law is: uk=-FFqxkqk, where Fq is component of integrator numerically modifying the control law in order to achieve a disturbance rejection in steady-state condition.

When the transient is over every state and additional variable dk become constant,

qk+1→q

q=q+y

y=0

which means we are able to obtain desired value y=0, rejecting the disturbance.

LMI method is used to implement this task.

Since we want to reject disturbance on linear displacement: Cy=[ 1 0 0 0].

Matrix Cz for LMI is built as follows:

Cz=100000000100000

The first row represents the output to be monitored and 1 in second row refers to the state of integrator, which is highlighted in order the regulation to be effective.

Initial state is taken as dx=[0;0;0;0;0.5], where 0.5 is the state of integrator.

After time instant t=15s, we introduce a constant disturbance with amplitude 1.

Figure 2.11 Disturbance rejection

As you can see, the integrator was able to recover the original output signal.

2.5.1 Position Tracking and Disturbance Rejection

By a slight modification, it is possible to track the reference. Let’s add a constant signal r=0.1 and keep everything from the previous section. The result is following:

Figure 2.12 Position tracking and disturbance rejection

In this case, integrator successfully rejects the disturbance, recovering the output signal when the transient is over and tracks the given reference.

Conclusion

In the work presented, the different optimal control techniques learned in class were

applied to a non-linear system whose mathematical model and parameters are known

physical, which is formed by platform able to rotate around a horizontal axis on which a mass slide, where the controlled variable were the displacement of the mass and angle of platform with respect to horizontal axis. To do this, first of all, the model was linearized around given equilibrium point, which makes it possible to use more robust methodologies designed for this type of system, after that, its stability and structural properties were analyzed. Having done

this, the methodologies were implemented and the response of the system was analyzed

subject to different disorders in the states in which it was possible to verify the effectiveness of

algorithms in determining the optimal control moves.

Bibliography

- Casavola, A. (2021). Notes from the System Theory course. University of Calabria, Department of Computer Engineering, Modeling, Electronics and Systems.

- Famularo, D. (2022). Notes from the course of Optimal Control. University of Calabria, Department of Computer Engineering, Modeling, Electronics and Systems.